# Representative Asset Assessment Examples

Operational Sense | Illustrative examples only

The asset contexts and IDs in this document are assumed for illustration and must not be represented as real plant data. Full scoring rationales are in the companion workbook.

Status: DRAFT PROPOSAL — weights, thresholds, default time bands, and override rules require approval and site calibration. Illustrative examples are not site data.

# Assessment results

| assessment_id | asset_id | equipment_tag | class_id | type_id | service_context (ASSUMED) | score_safety | score_production | score_environment | score_dependency | score_maintenance | weighted_score | criticality_index | band_rank | band_category | override_floor_rank | override_rules_triggered | final_rank | final_category | override_changed_category | data_origin |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CRA-0001 | AST-0001 | P-101 | EQ-PUMP | TY-PUMP-CENT | Cooling-water circulation pump; single duty, no installed standby; serves the main process line | 3 | 4 | 2 | 4 | 2 | 3.1 | 62 | 3 | High | 0 | None | 3 | High | No | Synthetic Scenario |
| CRA-0002 | AST-0004 | P-102 | EQ-PUMP | TY-PUMP-CENT | Boiler feed-water pump; no installed spare | 5 | 5 | 2 | 5 | 4 | 4.4 | 88 | 4 | Critical | 4 | R1 R3 | 4 | Critical | No | Synthetic Scenario |
| CRA-0003 | AST-0007 | P-201 | EQ-PUMP | TY-PUMP-CENT | Sump drainage pump; installed standby with automatic changeover | 1 | 1 | 2 | 1 | 1 | 1.15 | 23 | 1 | Low | 0 | None | 1 | Low | No | Synthetic Scenario |
| CRA-0004 | AST-0012 | P-305 | EQ-PUMP | TY-PUMP-CENT | Transfer pump for flammable solvent; standby installed; bunded area | 5 | 2 | 4 | 2 | 2 | 3.2 | 64 | 3 | High | 4 | R1 R3 R4 | 4 | Critical | Yes | Synthetic Scenario |
| CRA-0005 | AST-0002 | M-101 | EQ-MOTOR | TY-MOTOR-IM3 | Drive motor of P-101 (same cooling-water duty) | 3 | 4 | 1 | 4 | 3 | 3.1 | 62 | 3 | High | 0 | None | 3 | High | No | Synthetic Scenario |
| CRA-0006 | AST-0008 | M-301 | EQ-MOTOR | TY-MOTOR-IM3 | Conveyor drive motor on a single line; spare motor held in store | 3 | 3 | 1 | 3 | 2 | 2.55 | 51 | 2 | Medium | 0 | None | 2 | Medium | No | Synthetic Scenario |
| CRA-0007 | AST-0009 | M-402 | EQ-MOTOR | TY-MOTOR-IM3 | Workshop ventilation fan motor, general area | 1 | 1 | 1 | 1 | 1 | 1 | 20 | 1 | Low | 0 | None | 1 | Low | No | Synthetic Scenario |
| CRA-0008 | AST-0003 | C-101 | EQ-COMP | TY-COMP-SCREW | Instrument-air compressor; single unit; plant instruments depend on it | 4 | 5 | 2 | 5 | 4 | 4.1 | 82 | 4 | Critical | 3 | R3 | 4 | Critical | No | Synthetic Scenario |
| CRA-0009 | AST-0010 | C-102 | EQ-COMP | TY-COMP-SCREW | General-service plant-air compressor; duty/standby pair | 2 | 2 | 2 | 2 | 3 | 2.15 | 43 | 2 | Medium | 0 | None | 2 | Medium | No | Synthetic Scenario |
| CRA-0010 | AST-0011 | C-201 | EQ-COMP | TY-COMP-SCREW | Workshop compressor for intermittent tool use | 2 | 1 | 2 | 1 | 2 | 1.6 | 32 | 1 | Low | 0 | None | 1 | Low | No | Synthetic Scenario |

# Score rationale

| assessment_id | asset_id | factor | score | rationale (assumed context) |
| --- | --- | --- | --- | --- |
| CRA-0001 | AST-0001 | F1 Safety (personnel and equipment) | 3 | Loss of cooling can overheat connected equipment; hot-surface injury potential is moderate. |
| CRA-0001 | AST-0001 | F2 Production loss and downtime | 4 | Cooling loss forces a line stop; restart and recovery assumed to take more than a day. |
| CRA-0001 | AST-0001 | F3 Environmental impact | 2 | Water leakage is minor and drains on site. |
| CRA-0001 | AST-0001 | F4 Operational dependency and business continuity | 4 | No standby; several users share the cooling loop. |
| CRA-0001 | AST-0001 | F5 Maintenance and repair consequences | 2 | Routine centrifugal pump repair; spares assumed in stock. |
| CRA-0002 | AST-0004 | F1 Safety (personnel and equipment) | 5 | Loss of feed water can lead to low boiler level, a credible pressure-equipment hazard (assumed site context). |
| CRA-0002 | AST-0004 | F2 Production loss and downtime | 5 | Loss of steam supply stops the plant; prolonged outage if the boiler is damaged. |
| CRA-0002 | AST-0004 | F3 Environmental impact | 2 | Hot-water release is minor and contained. |
| CRA-0002 | AST-0004 | F4 Operational dependency and business continuity | 5 | Sole feed to steam supply; no workaround. |
| CRA-0002 | AST-0004 | F5 Maintenance and repair consequences | 4 | High-pressure multistage pump; long-lead spares and specialist repair assumed. |
| CRA-0003 | AST-0007 | F1 Safety (personnel and equipment) | 1 | No credible injury from failure. |
| CRA-0003 | AST-0007 | F2 Production loss and downtime | 1 | Standby takes over; no production effect. |
| CRA-0003 | AST-0007 | F3 Environmental impact | 2 | Minor overflow of drainage water on site possible. |
| CRA-0003 | AST-0007 | F4 Operational dependency and business continuity | 1 | Automatic standby; no dependency. |
| CRA-0003 | AST-0007 | F5 Maintenance and repair consequences | 1 | Simple repair with stock spares. |
| CRA-0004 | AST-0012 | F1 Safety (personnel and equipment) | 5 | Seal failure could release flammable liquid; credible fire potential. |
| CRA-0004 | AST-0012 | F2 Production loss and downtime | 2 | Standby covers the duty after a short disruption. |
| CRA-0004 | AST-0012 | F3 Environmental impact | 4 | Release could exceed local containment and be reportable (assumed). |
| CRA-0004 | AST-0012 | F4 Operational dependency and business continuity | 2 | Standby with manual changeover. |
| CRA-0004 | AST-0012 | F5 Maintenance and repair consequences | 2 | Seal and routine repair; spares assumed available. |
| CRA-0005 | AST-0002 | F1 Safety (personnel and equipment) | 3 | Electrical and hot-surface hazards are moderate. |
| CRA-0005 | AST-0002 | F2 Production loss and downtime | 4 | Motor failure causes the same loss of cooling as pump failure. |
| CRA-0005 | AST-0002 | F3 Environmental impact | 1 | No release from a motor failure. |
| CRA-0005 | AST-0002 | F4 Operational dependency and business continuity | 4 | Same dependency as the driven pump. |
| CRA-0005 | AST-0002 | F5 Maintenance and repair consequences | 3 | Rewind or replacement takes longer than a pump repair. |
| CRA-0006 | AST-0008 | F1 Safety (personnel and equipment) | 3 | Moving-machinery hazard on the conveyor; medical-treatment injury possible. |
| CRA-0006 | AST-0008 | F2 Production loss and downtime | 3 | Line stops for part of a day while the spare is fitted. |
| CRA-0006 | AST-0008 | F3 Environmental impact | 1 | No release. |
| CRA-0006 | AST-0008 | F4 Operational dependency and business continuity | 3 | Partial backup through the stored spare. |
| CRA-0006 | AST-0008 | F5 Maintenance and repair consequences | 2 | Change-out within a shift using the stored spare. |
| CRA-0007 | AST-0009 | F1 Safety (personnel and equipment) | 1 | No credible injury. |
| CRA-0007 | AST-0009 | F2 Production loss and downtime | 1 | No production effect. |
| CRA-0007 | AST-0009 | F3 Environmental impact | 1 | No release. |
| CRA-0007 | AST-0009 | F4 Operational dependency and business continuity | 1 | No dependency. |
| CRA-0007 | AST-0009 | F5 Maintenance and repair consequences | 1 | Simple, low-cost replacement. |
| CRA-0008 | AST-0003 | F1 Safety (personnel and equipment) | 4 | Loss of instrument air can send control valves to fail positions; serious process upset possible. |
| CRA-0008 | AST-0003 | F2 Production loss and downtime | 5 | Air loss trips the plant; recovery is prolonged. |
| CRA-0008 | AST-0003 | F3 Environmental impact | 2 | Compressor oil leak is minor. |
| CRA-0008 | AST-0003 | F4 Operational dependency and business continuity | 5 | Single air supply; no backup. |
| CRA-0008 | AST-0003 | F5 Maintenance and repair consequences | 4 | Air-end overhaul needs specialist work and long-lead parts. |
| CRA-0009 | AST-0010 | F1 Safety (personnel and equipment) | 2 | Pressurised-air and hot-surface hazards are minor. |
| CRA-0009 | AST-0010 | F2 Production loss and downtime | 2 | Standby covers most demand; short reduction in rate. |
| CRA-0009 | AST-0010 | F3 Environmental impact | 2 | Oil leak is minor and contained. |
| CRA-0009 | AST-0010 | F4 Operational dependency and business continuity | 2 | Standby with manual changeover. |
| CRA-0009 | AST-0010 | F5 Maintenance and repair consequences | 3 | Repairs take 1 to 2 days; spares from supplier. |
| CRA-0010 | AST-0011 | F1 Safety (personnel and equipment) | 2 | Pressurised-air hazard is minor. |
| CRA-0010 | AST-0011 | F2 Production loss and downtime | 1 | No production effect. |
| CRA-0010 | AST-0011 | F3 Environmental impact | 2 | Minor oil leak possible. |
| CRA-0010 | AST-0011 | F4 Operational dependency and business continuity | 1 | No dependency; intermittent use. |
| CRA-0010 | AST-0011 | F5 Maintenance and repair consequences | 2 | Repair within a shift. |

The worked example in the framework document describes CRA-0004, a flammable-solvent transfer pump with standby. The weighted score is 3.20, index 64.0 (High before overrides); the safety override raises the final category to Critical. Refer to the workbook formulas for reproducible calculations.