# Criticality Classification Guidelines

Operational Sense | Asset Criticality Assessment Framework v1.0

Use this guide with Asset_Criticality_Scoring_Matrix.xlsx. Classification bands and overrides are proposed rules, not an approved site standard.

Status: DRAFT PROPOSAL — weights, thresholds, default time bands, and override rules require approval and site calibration. Illustrative examples are not site data.

# Classification rules

| A. Category bands (applied to the criticality index, rounded to 1 decimal) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Category | Index range | Equivalent weighted score | Meaning | Suggested use (proposal, not a maintenance policy) |
| Critical | 80.0 to 100.0 | 4.0 to 5.0 | Failure has severe consequences for people, environment, production or continuity. | Highest priority for condition monitoring, spares review and preventive-maintenance review. |
| High | 60.0 to 79.9 | 3.0 to 3.99 | Failure has major consequences. | Scheduled preventive maintenance and routine monitoring. |
| Medium | 40.0 to 59.9 | 2.0 to 2.99 | Failure has moderate consequences. | Standard preventive maintenance. |
| Low | 20.0 to 39.9 | 1.0 to 1.99 | Failure has limited consequences. | Basic checks; run-to-failure may be acceptable where spares are easy to get. |
| B. Override rules (final category = the higher of the band category and the override floor) |  |  |  |  |
| Rule | Condition | Effect | Reason |  |
| R1 | Safety score >= 5 | Final category = Critical | A credible fatality or major release must not be diluted by low scores elsewhere. |  |
| R2 | Environmental score >= 5 | Final category = Critical | A major or off-site release must not be diluted. |  |
| R3 | Safety score >= 4 | Final category at least High | Serious-injury potential must not fall below High. |  |
| R4 | Environmental score >= 4 | Final category at least High | Reportable release potential must not fall below High. |  |
| C. Assignment procedure |  |  |  |  |
| Step | Action |  |  |  |
| 1 | Identify the asset instance (asset_id, tag, type) and describe its service and any standby or dependants. |  |  |  |
| 2 | Score each of the 5 factors from 1 to 5 using sheet Scoring_Scales; write a short rationale for each score (mandatory for scores of 4 or 5). |  |  |  |
| 3 | Weighted score = sum of (factor score x weight). |  |  |  |
| 4 | Criticality index = weighted score x 20 (range 20 to 100). |  |  |  |
| 5 | Assign the band category from the index. |  |  |  |
| 6 | Apply override rules R1 to R4; final category = higher of band and floor. |  |  |  |
| 7 | Record assessor, date, status and versions; have operations, safety and maintenance review before approval. |  |  |  |
| D. Boundary, ties and reassessment |  |  |  |  |
| Topic | Rule |  |  |  |
| Boundaries | Thresholds apply to the index rounded to 1 decimal. An index of exactly 60.0 is High; exactly 80.0 is Critical; exactly 40.0 is Medium. |  |  |  |
| Uncertain scores | Choose the higher level and record why; do not average two levels. |  |  |  |
| Driver and driven equipment | Score each asset on its own consequences. A motor or coupling serving a driven machine normally shares the production and dependency scores of that machine. |  |  |  |
| Reassessment | At least every 12 months (proposed), and after a process change, a change in standby or redundancy, a serious incident, or a change in weights or bands. |  |  |  |
| Inflation check | If most assets land in Critical or High, review scale use; the method must differentiate. |  |  |  |

# Parameters

| Parameter | Value | Note |
| --- | --- | --- |
| Weight - F1 Safety | 0.3 | Proposed. Highest weight: safety harm is not recoverable. |
| Weight - F2 Production loss and downtime | 0.25 | Proposed. |
| Weight - F3 Environmental impact | 0.15 | Proposed. |
| Weight - F4 Operational dependency and continuity | 0.15 | Proposed. |
| Weight - F5 Maintenance and repair consequences | 0.15 | Proposed. |
| Total weight | 1 | Must equal 100%. |
| Weights check | OK |  |
| Band threshold - Critical (index >=) | 80 | Equals weighted score 4.0. |
| Band threshold - High (index >=) | 60 | Equals weighted score 3.0. |
| Band threshold - Medium (index >=) | 40 | Equals weighted score 2.0. Below this = Low. |
| Override R1 - Safety score >= (forces Critical) | 5 | Proposed. |
| Override R2 - Environmental score >= (forces Critical) | 5 | Proposed. |
| Override R3 - Safety score >= (minimum High) | 4 | Proposed. |
| Override R4 - Environmental score >= (minimum High) | 4 | Proposed. |

# Operational review guidance

Reassess at least every 12 months (proposed), and after process changes, changes to standby arrangements, serious incidents, or changes to weights/bands.

Assess the consequence of losing the asset function in normal operation. Count only standby, protection, and workarounds that actually exist.

Keep asset criticality separate from failure-case severity, asset condition, and failure likelihood.

If many assets are rated Critical or High, review scoring consistency and whether the scales distinguish asset duties appropriately.

Use site-specific production and repair-time bands where available; document rationale and assumptions.